Club Data Corporation

10125 Crosstown Circle		(612) 941-0855

Suite 115		(612) 941-7514 fax

Eden Prairie, MN 55344		email: clbdata@ibm.net

Windows 95 - Installing a larger hard drive.

1.	Install your new drive as a slave, setting the jumpers on both drives appropriately. Make sure your CMOS knows there’s a second hard disk attached.

2.	Start Win95 and then choose the MS-DOS prompt from the start menu (or desktop icon if you have one).

3.	Run FDISK to set up your partitions on the new drive. (Be careful to select the 2nd hard disk, not your good one!)

4.	Format the partitions on the new drive, using the /S option on the partition you will be booting from.

5.	Format a bootable floppy and copy FDISK from the \WINDOWS\COMMAND directory. (If you already have a Win95 boot diskette, you can skip this step since it has the proper files already on it.)

6.	Close the DOS session, close Windows and restart the computer so that the new hard disk is recognized by not only the PC but by Windows, too.

7.	When the system restarts, click START, PROGRAMS, MS-DOS Prompt. (This does NOT work if you restart in MS-DOS mode – you must get to a DOS prompt from the Start menu)

8.	From the MS-DOS prompt type: (using the correct drive letters) xcopy c:\*.*  d:  /c /e /h /k /r

9.	Shut down and power off. Reconfigure your drives as desired - e.g. remove the old drive and jumper the new one to work as a stand alone.

10.	Start Win95.

Note 1: It may happen that the new drive won’t boot because the new hard disk does not have an active partition. If this occurs, use the bootable floppy from step 5, run FDISK and set the bootable partition to be active. Then reboot the hard disk again and everything should be okay.

Note 2: When the instructions say start Win95 and then go to a DOS prompt, don’t try booting directly to a DOS prompt. This XCOPY procedure requires the 32 bit version of XCOPY which is only active  from a DOS box within Win95: start Win95 normally and then get to a DOS prompt from the start menu.